Pool Safety Operating Procedures

Trinity Trainer Pool

Greys Hill

Henley-on-Thames

This document is produced in accordance with the guidance given in HSG179 (Fourth edition) Published 2018 and contains the following information:

Normal Operating Procedures (NOP)			Pages 2 - 7

Emergency Action Plan (EAP) 				Pages 8 – 11

Risk Assessments 						Pages 12 – 16

Pool rules							Page 17

| Version 1 | May 2007 |  |
| --- | --- | --- |
| Version 2 | October 2009 |  |
| Version 3 | January 2012 |  |
| Version 4 | April 2013 |  |
| Version 5 | July 2014 | Review date July 2015 |
| Version 6 | July 2015 | Review date July 2016 |
| Version 7 | November 2016 |  |
| Version 8 | November 2017 |  |
| Version 9 | November 2018 |  |
| Version 10 | November 2019 |  |
| Version 11 | February 2020 |  |
| Version 12 | November 2021 |  |
| Version 13 | December 2022 |  |
| Version 14 | January 2025 |  |

Trinity Trainer Pool is a registered charity (Registered number 286435 ) and is managed by a committee of volunteers. The building is owned by Oxfordshire County Council.

1. DETAILS OF THE SWIMMING POOL AND BUILDING

1.1 The pool

A new pool was installed by Purley Pools in August 2004. It is an indoor pool and of an above ground construction. It is 0.75metres deep, 9.5 metres long and 3.5 metres wide (Total area= 33.25m). The pool is lined with a heavy duty butyl liner which was replaced in May 2020. The flooring surrounding the pool, shower and toilet areas was renewed in August 2009 with Tact tiles which are non-slip tiles specifically for use in wet areas.

The pool is entered via purpose made steps with handrails on both sides. The pool has a plastic footbath which is refilled with water from the pool by each party using the pool. It is emptied after each use and the footbath left to drain by the access steps.

When not in use the pool is covered by a purpose made cover suspended on a roller at the end of the pool. Due to the temperature of the water combined with the action of the chemicals, the cover only lasts about two years before starting to break up. The cover was replaced in 2024.

The water level in the pool is topped up automatically by an auto top-up system installed in May 2020.

1.2 The building

The building is an old building, formally Trinity School. It is accessed via a single entrance which is locked when the pool is not in use and the keys are kept in a key safe which is locate next to the entrance door and is accessed through a code which is changed regularly.

From the single entrance there is a hallway giving direct access to the pool area straight ahead. To the right of the hall there a door giving access to a corridor which has male and female changing rooms off  it. From this corridor there is a second doorway giving access to the pool side.  The public are asked to remove outdoor shoes beyond this point and there are notices to that effect.

Toilets and showers are to the left of the hall but are accessed by a door from the main pool hall.

The boiler room is situated in the left hand corner of the main pool hall. It is a partitioned construction accessed via a door which is kept locked.

New showers were installed in August 2004. Shower area was fitted with specialized shower boarding in 2012 The inside of the building was repainted and decorated in August 2024. The entrance hall, both changing rooms, the boy’s toilets and large areas of the pool hall were lined with specialised boarding in March 2021.  This has significantly reduced the amount of redecorating needed.  Any areas needing repainted are done as and when required.

  - Pool plant

The pool water is heated by a boiler situated in the boiler room. A new Worcester Bosch wall mounted boiler with heat exchanger was installed in August 2013. The desired temperature is 31-33oC (88-92F). The water temperature is adjusted by the temperature controller on the back wall of the plant room. The boiler is serviced annually by Henley Heating and Plumbing and a gas safety certificate issued.

The time clock controls the boiler in order to maintain adequate temperature; the water is heated approximately 10 hours in every 24hrs.

The circulation pump runs continuously and takes approximately 6 hours to circulate the water once

1.4 The boiler room

The boiler room contains the following equipment.

- Sand filter – Triton TR 100 (major repairs carried out Oct 2021)

- UV filter fitted in early 2024

- Water pump – on floor  (pump new August 2018)

- Boiler – Worcester Bosch

- Time clock and temp controller on back wall of plant room.

- Mains water stopcock – On the far left wall behind the sand filter

- Electric boiler for water to showers and basins

1.5 Heating of premises, showers and hot water taps.

The main pool hall, toilets, shower area and the changing room are heated by Oxfordshire County Council (plant in basement underneath Social Services Office)

The showers and basins are heated by an independent electric boiler situated in the plant room.

2. POTENTIAL RISKS

The risks of providing a pool facility have been assessed and the risks arising from the unique features of this pool have been identified. The detailed requirements included within these Pool Safety Operating Procedures have been formulated to ensure control and/or reduction in the risks present.

See Appendix 1 for full details of risk assessment and controls

  - Pool Hazards

The pool is covered by a plastic cover when not in use. This must be fully rolled back off the pool before anyone enters the pool. There are notices to this effect.

As the pool is an over ground pool the sides are high from the ground. Pool users are not allowed to sit on the edge of the pool or stand on the sides of the pool to jump in. There are notices to this effect.

2.2 Flooring in area surrounding the pool and in the shower and changing areas

The floor area surrounding the pool and in the shower and changing room areas can become very wet and potentially slippery. The flooring material around the pool and in the shower and toilet areas is slip resistant. There is slip resistant flooring in the changing room area.  Pool users are forbidden to run on the pool side and changing rooms. Parents are asked to supervise children in the shower area so that this area does not become excessively wet.

  - Radiator and hot water pipes

Large heating pipes run from the building next door through the pool building at floor level. These sometimes become very hot. The temperature is controlled by Oxfordshire County Council. To reduce the risk of burning these pipes have been covered with purpose made covers with vents to allow the heat to come through. There are notices asking people not to sit or stand on these covers. There are hazard notices above the radiators.

  - Steps from changing area to pool hall

There is a steep step up and down from the changing area into the main pool hall and from the pool hall into the toilet and shower area. These steps are highlighted with hazard tape. There are warning notices highlighting the hazard.

  - Steps into the swimming pool

The steps for entering into and getting out of the pool are purpose made for use with above ground pools and have hand rails and anti-slip surface.

2.6 Chemicals

Strong chemicals are used to purify the water; these are controlled to normal limits. Chemicals are stored in a locked cupboard and are only used by authorized personnel. The chemicals used are tablet format and granules to reduce risk of skins or eye contact and subsequent chemical burns. Gloves and goggles are provided. The first aid box contains eye wash should any user get chemicals into their eyes.

  - Hot taps and showers in the shower/toilet area

The showers and hot taps are fitted with thermostatically controlled valves.

3. DETAILED WORK INSTRUCTIONS

3.1 Access to the pool

Access to the pool building is controlled by the keys being stored in a key safe situated near the door. The access code is only given to those authorized to use the pool.

3.2 Inspection of the pool before swimming starts

Before any swimmers enter the pool the designated adult in charge should make a visible check of the building and the pool to ensure all health and safety measure outlined in this document are in place.

3.3 Removal of the pool cover

The designated adult in charge must make sure that the pool cover is fully retracted before any swimmers enter the pool and that the cover is replaced after use. Before replacing the cover a visible check of the pool should be made to ensure all swimmers are out of the pool.

3.4 Supervisory arrangements

The maximum number of people allowed in the pool at any one time is 12.

In a teaching situation the maximum ratio is one teacher to twelve pupils (12:1)

For Adult and baby or child lessons the max ratio is 6 pairs :1 teacher.

For private pool hire the ratio of at least one adult to two children. There needs to be a minimum of 2 adults present in the building at all times when hiring the pool.

The designated responsible adult in charge must remain in the pool area at all times and ensure that all swimmers are out of the building before he/she leaves the pool area.

All teachers must have the minimum lifesaving qualification – Royal Life Saving Society Rescue Award for Swimming Teachers and Coaches

3.5 Behaviour

All pool users must abide by the pool rules set out in appendix 2 and displayed in the pool area and changing rooms.

3.6 Swimming aids

There are a number of swimming aids available at the pool side. Floats are stored on the ledge behind the pool. Woggles are stored hanging on the pool hall wall at the end of the pool. Swimming aids are checked for wear and tear on a regular basis and damaged equipment removed as necessary. Before using swimming aids the designated responsible adult must check that they are in good condition. All pool users must ensure that swimming aids are put back in their designated storage area after use.

4. FIRST AID EMERGENCY EQUIPMENT SUPPLY

First aid equipment is located in the main pool hall and includes a first aid box, resuscitation mask and pool evacuation box including foil blankets.First aid box contents totally renewed Jan 2025

In emergency situation 999 should be called.

The rescue equipment is located on the wall in the main pool wall and includes a reach pole and a rescue ring and rope.

Pool users are required to have a charged mobile phone available to call emergency services. The signal and level of charge must be checked prior to using the pool.

5. DETAILS OF MAINTENANCE ARRANGEMENTS

5.1 General cleaning of the pool building

The entire pool building, showers, basins and toilets are cleaned daily. Pool users are asked to leave the building clean and tidy after use. Soiled nappies should be taken away.

5.2 Maintenance of the pool

Strong chemicals are used to purify the water; these are controlled to normal limits. The chemical levels are also checked daily and levels are recorded on the record sheets in the plant room. In addition the pool is serviced weekly basis by Aztec Services, Henley on Thames. Should any user experience any form of irritation, or notice unusually strong chlorine-type smells the procedures outlined in the Emergency Action Plan should be followed.

Chemicals used

Chlorine tablets

Chlorine granules

pH plus – added to raise pH levels

Water clarifier.

Shock dose chemical

Jolly Gel pool flocculent.

Bacteriological tests of the pool water are carried out monthly by Rochester Midland Corporation. The mains water is tested for legionella annually.

5.3 Boiler maintenance

The boiler that heats the pool is serviced annually by Henley Heating and Plumbing and a gas safety certificate issued.

A new wall mounted  Worcester condenser boiler for heating the pool water was installed in August 2013.

5.3 Electrical safety

Electrical safety inspection is carried out annually and a safety certificate issued.

5.4 Cleaning and disinfecting shower heads

The shower heads are descaled and disinfected regularly.

6. CONDITIONS OF HIRE

6.1  Swimming teachers and schools hiring the pool

Any person offering swimming lessons at Trinity Trainer pool must:

- Must use the pool in accordance with the Pool Safety Operating Procedures

- Must-

- Be fully qualified swimming instructors

- have been DBS enhanced level checked in the last three years

- hold as a minimum Royal Life Saving Society Rescue Award for Swimming Teachers and Coaches

- hold personal and public liability insurance

- take full responsibility for the group while in the pool and any other part of the building

- report any problems with the pool or equipment to a committee member

- ensure the cover is replaced on the pool after use

- ensure that the main pool hall, the changing rooms, showers and toilet areas are left clean and tidy

- take responsibility for the safe keeping of the key issued to them and ensure that it is not given to any unauthorized person

- Ensure that lights are turned off, windows are closed and the building is locked when leaving

6.2  General pool hire by the public

The pool may be hired by individuals or groups. Private hire is only available through the pool website and hirers must agree to abide by the pool rules and pool safety operating procedures. The hirer must be in attendance during all bookings made by them. By making a booking, the hirer accepts full responsibility for all users in their group and agrees to the pool rules accepting that there will be no lifeguard present and that Trinity Trainer Pool Association accepts no responsibility for any incidents during private hire. The key safe code will be sent out automatically after a booking is made. The code must not be given to anyone else. Trinity Trainer Pool Association has Public Liability cover in place for the pool. This does not cover the pool hirer or anyone in their group/party should injury be caused through their own negligence.

The designated responsible adult must:

- take full responsibility for the group while in the pool and any other part of the building

- ensure a ratio of at least one adult to two children

- read and abide by all notices

- report any problems with the pool or equipment to a committee member

- ensure the cover is replaced on the pool after use

- ensure that they do not leave the pool hall until all other users have got out of the pool. They must check that all members of their group/party have left the building before locking the door.

- ensure that the main pool hall, the changing rooms, showers and toilet areas are left clean and tidy

- take responsibility for the safe keeping of the key issued to them and ensure that it is returned to the key safe immediately after use

- ensure that lights are turned off, windows closed and the building is locked when leaving

EMERGENCY ACTION PLAN

Introduction

The following procedures are set out so that all pool hirers are able to react to an emergency situation with confidence and efficiency so that the effects of an emergency situation on pool users can be controlled and minimized.

All pool hirers have a statutory responsibility under the Health and Safety at Work Act (1974) to have read and learnt these procedures before using the pool or taking responsibility for others (primarily children) using the pool.

Overcrowding

The maximum number of people allowed in the pool is 12.

In a teaching situation the maximum ratio is one teacher to twelve pupils (1:12)

For Adult and baby or child the max ratio is 1:6 pairs.

For private pool hire the ratio is at least one adult to two children

For private hire there should be at least 2 adults present in the building at all times.

Disorderly behavior

All swimmers must behave in an orderly fashion and obey the pool users rules outlined in appendix 2 and displayed in the changing rooms and on the wall of the main pool hall. Parents are required to supervise children at all times particularly siblings of those taking part in swimming lessons. Any person who fails to abide by the rules will be asked to leave the pool area.

Lack of water clarity

Water clarity – The designated adult in charge should check the clarity of the water before anyone enters the pool. If the bottom of the pool is not clearly visible then the pool should not be used and this should be reported to the booking secretary or a committee member

Water Contamination

This is likely to be caused by vomit, faecal fouling or blood getting in the water.

Solid stool

- Immediately evacuate everyone from the pool.

- Don disposable gloves available by the first aid box.

- Immediately remove stool from the pool using the bucket and fine mesh net and flush down the toilet.

- Report the incident immediately to the booking secretary by emailing hello@trinitytrainerpool.co.uk

- The booking secretary will ascertain any further action required depending on whether all the stool has been fully captured.

- The pool must be shock dosed and closed for 6hrs

- All equipment that has been used in this process should be disinfected using a 1% solution of hypochlorite.

- If the pool is operating properly with appropriate disinfectant residuals and pH values after 6 hrs, no further action is necessary.

Diarrhoea

- Immediately evacuate everyone from the pool. Instruct pool users to shower.

- Don disposable gloves available by the first aid box.

- Immediately remove stool from the pool using the bucket and fine mesh net and flush down the toilet.

- Report the incident immediately to the booking secretary by emailing hello@trinitytrainerpool.co.uk

- The booking secretary must close the pool to further use for a period of at least 24hr

- Shock dose the pool with one sachet of pool shock

- Put a dose of Jolly Gel into pump filter

- Hold the disinfectant residual at the top of its set range i.e. 3.0mg/l and the pH value at the bottom of its range (eg pH 7.0-7.2). This will maintain the normal level of microbiological protection.

- Vacuum any obvious retained faeces

- The skimmer basket must be cleaned and a backwash carried out for a minimum of three minutes.

Before reopening a full chemical test must be carried out to establish safe water quality.

Blood and vomit

For small amounts of blood such as a minor nose bleed, remove the casualty from the pool immediately and remove the blood from the pool using the red bucket.

For large amounts of blood or vomit treat as per diarrhoea.

Outbreak of fire

In the event of a fire the designated responsible adult must order the pool to be evacuated. All pool users must leave the building immediately via the main door, which is clearly labeled as a fire exit, without going to the changing rooms to collect personal belongings. There is an emergency evacuation box kept in the entrance hall which contains foil blankets to be provided to swimmers who have had to be evacuated to outside. The designated responsible adult should pick up the box on the way out of the building. The designated meeting point is by the steps in the corner of the car park. Once the building has been evacuated call 999 from a mobile phone and inform member of the committee.

There are four fire extinguishers, one hydrospray in the entrance hall, one in the changing room area, one C02 in the main pool hall and one water in the shower area. Extinguishers are serviced annually by Chubb. These should only be operated if it is safe to do so.

Bomb threat

Should a bomb threat be received the pool should be evacuated in accordance with the fire evacuation procedures.

Lighting failure

In the event of normal lighting failure, emergency lighting will come on automatically in the main pool hall (the emergency lighting is tested once a week using the test switch which is wall mounted in the entrance hall on the left just before you go through the double doors into the main pool hall.) The designated person in charge must order the pool to be evacuated immediately. Swimming should not be resumed until power is restored.

Structural Failure

In the event of structural failure the pool should be evacuated in accordance with the fire evacuation procedures.

Emission of toxic gases

If swimmers notice a smell of gas the adult in charge should immediately evacuate all users from the pool as per fire evacuation. The booking secretary or a member of the committee should be notified as quickly as possible. The pool will be closed and an out of order notice placed on the door until the situation has been investigated and resolved.

Serious injury to a bather

In the event of an accident, sudden illness or other incapacity involving a pool user, the designated responsible adult (i.e. swimming teacher or person hiring the pool) should take appropriate action.

For minor injuries there is a first aid kit on the table in the main pool hall.

In the event of a serious injury the pool should be evacuated and the designated responsible adult should perform an effective rescue. There is a safety hook with an extending handle hanging near the boiler room and a resuscitation mask with the first aid box.

If the event is life threatening the person in charge should stay with the victim and give first aid as appropriate. Another responsible adult should telephone the emergency services using their mobile phone.

When phoning, give the pool address as:-

TRINITY TRAINER POOL

GREYS HILL

HENLEY ON THAMES

RG9 1SJ

BETWEEN TRINITY CHURCH GRAVE YARD AND SACRED HEART SCHOOL

If the person attending to the casualty does not require assistance, the second person or a third responsible adult should supervise remaining children in the changing room.

All accidents whether minor or more serious MUST be recorded in the accident book which is the cupboard in the pool entrance hall.

Injuries to persons working at the pool

All serious injuries sustained at the pool and requiring hospital treatment or more than seven days off work for self employed or school employed personnel MUST be reported to the booking secretary either in writing or by e-mail as soon as possible

The following information must be provided:

Name of the injured party

Address of the above

Contact telephone number

Age

Cause of accident

A brief description of the events leading to the accident, any action taken and by whom.

This information is required under the Reporting of Injuries Diseases and Dangerous Occurrence Regulations 1995 (RIDDOR). Failure to do so could lead to the cancellation of any further use of the pool.

Accidents to people other than workers

Accidents to members of the public or others who are not at work (such as customers or volunteers) must be reported if:

- they involve work activity

- they result in an injury

- the person is taken directly from the scene of the accident to hospital for treatment to that injury

Discovery of a casualty in the water

The adult in charge should quickly clear the pool and perform an effective rescue of the swimmer. Carry out emergency procedures as for serious injury to a bather.

Risk Assessments

Date:  March 2013, updated Nov 2019, November 2021, Jan 2025.

Key: Likelihood/Severity	  1= Low	2 = Medium	3 = High  		 Risk = Likelihood x Severity

| Activity/element | Hazard | Who might be effected | Risk Analysis |  |  | Can the risk be eliminated | Controls in place to reduce the risk |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Likelihood | Severity | Risk |  |  |
| Access to the pool | Unauthorized people may access the pool resulting in unsupervised swimming | Unauthorized users | 1 | 3 | 3 | No | Key to the external door is kept in a key safe by the door into the building. The access code is changed regularly. The code is only given to authorized users |
| Steps from the entrance hall into the main pool hall | Fall or trip | All pool users | 1 | 2 | 2 | No | The step has hazard warning tape and there are signs to highlight the step |
| Steps from the corridor outside the changing room into the main pool hall | Fall or trip | All pool users | 1 | 2 | 2 | No | The step has hazard warning tape and there are signs to highlight the step. |
| Steps from the main pool hall into the shower/toilet area | Fall or trip | All pool users | 1 | 2 | 2 | No | The step has hazard warning tape and there are signs to highlight the step |
| The floor area surrounding the pool and in the shower and changing room areas can become very wet | Slip or fall | All pool users | 2 | 2 | 4 | No | The floor covering is anti-slip material. There are signs to warn of the hazard |
| Using the pool | Drowning | All pool users particularly children | 2 | 3 | 6 | No | All users to follow safety guidelines within The Pool Safety Operating Procedures. All teachers to hold as a minimum the Royal Life Saving Society Rescue test for teachers and coaches. Pool hirers and families to follow rules laid out in the contract for hire document |
| Steps getting into and out of the pool | Fall from a height | All pool users particularly children | 2 | 3 | 6 | No | The steps are specifically designed for the purpose of entering an above ground pool. They have anti slip surface and a hand rail. Only one user at a time is allowed to use the steps |
| The pool is an above ground pool and the sides are high | Fall from a height | All pool users particularly children | 2 | 3 | 6 | No | Pool users are not allowed to sit or stand on the edge of the pool and are not allowed to jump in. There are signs to this effect and it is outlined in the hirers contract. |
| Pool water | Risk of Cryptosporidium infection | All pool users | 1 | 3 | 3 | No | Chemicals are used to disinfect the pool water and there is a UV filter in place. Chemical levels are tested on a daily basis and the pool is serviced by Aztec once a week. The pool water is tested for bacteria monthly by Rochester Midland Corporation. |
| Pool cover | Entrapment in the pool | All pool users | 1 | 3 | 3 | No | Pool cover has to be rolled back before pool is used. There are signs to this effect |
| Radiator and hot water pipes | Burning | All pool users particularly children | 2 | 2 | 4 | No | Pipes are boxed in and there are signs asking people not to sit on the boxes. There are warning signs above the radiators. Parents are asked to supervise children at all times |
| Hot taps and showers in the changing room | Scalding | All pool users particularly children | 2 | 3 | 6 | No | Showers and basin taps are fitted with thermostatically controlled valves |
| Domestic water in showers and basins | Legionella | All pool users | 1 | 3 | 3 | No | The showers are descaled regularly. The water is tested by Rochester Midland Corporation on an annual basis. During periods of low usage of the pool e.g. during summer holidays the water in the showers and basins is run regularly. |
| Glare from the windows causing reflection off the water. | Reduced visibility in the pool | All pool users | 2 | 3 | 6 | No | All teachers are made aware of this risk. New windows with tinted glass were fitted by Oxfordshire County Council in Spring 2014. |
| Chemicals | Skin irritation | All pool users | 2 | 2 | 4 | No | The chemical levels in the pool are tested daily and records are kept in a folder in the plant room. |
| Chemicals | Poisoning |  | 1 | 3 | 3 | No | Chemicals are stored in a locked cupboard. |
|  | Toxic fumes | Personnel using chemicals. Pool users |  |  |  | No | The material safety data sheet (MSDS) for each chemical used is printed out and available in the plant room. Emergency procedures are outlined in the Emergency Action Plan. |
| Chemicals | Splash to the eyes | Personnel using chemicals | 2 | 2 | 4 | No | There is protective clothing in the plant room for use when using chemicals- gloves, and eye protection. There is eye wash available in the plant room and in the first aid box. |

Appendix 2

Pool Rules

Swimmers must not enter the pool unless there is a designated responsible adult in the pool.

There must be no more than 12 people in the pool at any one time

The pool cover must be removed before anyone enters the pool.

People must not enter the pool building if they have had vomiting or diarrhoea until 48 hours after the last symptoms. This applies to adults taking children to swimming and any siblings of those swimming even if they are not entering the pool themselves.

Outdoor footwear must be removed in the entrance hall.

All users must blow their nose and have a shower before entering the pool. Parents are responsible for making sure that their children do so.

All users must wash their hands after using the toilet

If babies have soiled their nappies their bottoms must be thoroughly cleaned before they enter the pool. The person changing the nappy must wash their hands immediately.

All babies and children in nappies must wear designated swim nappies which must be freshly put on immediately before entering the pool.

Nappies must be taken away and NOT disposed of in the pool bins.

All open cuts or wounds must be covered.

Children must be supervised by an adult at all times.

No running in the changing rooms or pool hall.

No sitting, standing on, or jumping in from the pool edge.

No biting, spitting, hitting or ducking.

No food to be consumed in the pool hall. Chewing gum must be removed before swimming.

The pool and pool hall must be left clean and tidy with the pool cover put back on the pool before leaving.

Enjoy your swim